ETAS Integration for (F/X)ETK v1.0 beta

How to integrate the ETK driver and A2L adaption or all ETK features DRAFT

User Guide

Copyright

The data in this document may not be altered or amended without special notification from ETAS GmbH. ETAS GmbH undertakes no further obligation in relation to this document. The software described in it can only be used if the customer is in possession of a general license agreement or single license. Using and copying is only allowed in concurrence with the specifications stipulated in the contract.

Under no circumstances may any part of this document be copied, reproduced, transmitted, stored in a retrieval system or translated into another language without the express written permission of ETAS GmbH.

© Copyright 2025 ETAS GmbH, Stuttgart

The names and designations used in this document are trademarks or brands belonging to the respective owners.

ETAS Integration for (F/X)ETK’s | User Guide R01 EN – 01.2025 DRAFT

Contents

1	Introduction	5

1.1	Definitions and Abbreviations	5

1.2	Conventions	6

2	General usage of (F/X)ETK’s with different uC types	7

3	ETK driver structure	8

4	Handshake	9

4.1	Needed Files	9

4.2	Changes in Files for integration	10

4.2.1	CompileTime.c	10

4.2.2	ETK_Integration_Cfg.h	10

4.2.3	ETK_SER_Handshake.h	11

4.2.4	GetTime.h	11

4.3	Call of function	11

4.4	A2L configuration in XCT Tool	12

5	Measurement methods	13

5.1	Distab17 method	13

5.1.1	Needed Files	13

5.1.2	Changes in Files for integration	14

5.1.3	Call of function	15

5.1.4	A2L configuration in XCT Tool	16

5.1.5	Verify in XCT Tool and use in INCA	17

5.2	Trace based measurement method	19

5.2.1	Needed Files	19

5.2.2	Changes in Files for integration	19

5.2.3	Call of function	20

5.2.4	A2L configuration in XCT Tool	21

5.2.5	Verify in XCT Tool and use in INCA	21

5.3	Supplementary Distab measurement method	22

5.3.1	Needed Files	22

5.3.2	Changes in Files for integration	22

5.3.3	Call of function	23

5.3.4	A2L configuration in XCT Tool	24

5.3.5	Verify in XCT Tool and use in INCA	25

6	Calibration	27

6.1	Needed Files	27

6.2	Changes in Files for integration	27

6.2.1	CompileTime.c	27

6.2.2	ETK_Integration_Cfg.h	28

6.2.3	Call of function	28

6.2.4	A2L configuration in XCT Tool	29

6.2.5	Verify in XCT Tool and use in INCA	31

7	Advanced Code Check (ACC)	35

7.1	Needed Files	35

7.2	Changes in Files for integration	35

7.2.1	ETK_CodeCheck.c	35

7.2.2	ETK_Integration_Cfg.h	36

7.2.3	Call of function	36

7.2.4	A2L configuration in XCT Tool	37

7.2.5	Verify in XCT Tool and use in INCA	37

8	Coldstart measurement	39

8.1	Changes in Files for integration	39

8.1.1	ETK_SER_Coldstart.c	39

8.1.2	ETK_Integration_Cfg.h	40

8.1.3	Call of function	40

8.1.4	A2L configuration in XCT Tool	40

8.1.5	Verify in XCT Tool and use in INCA	40

9	Contact Information	42

Figures	43

Tables	44

Index	45

Revision History	46

# Introduction

## Definitions and Abbreviations

AML

ASAM-MCD 2MC (aka ASAP2) Meta Language

A2L

ASAM-MCD 2MC (aka ASAP2) Description Language

DISTAB

Display Table is the method used for Data Acquisition

ECU

Electronic Control Unit

EMEM

Extension Memory

ERAM

Emulation Random Access Memory

ETK Tool

Configuration Toolset used to configure the ETK, FETK and XETK devices

HWC

Hardware configuration from workspace in INCA

PD-device

Production device without trace unit and EMEM

RP

Reference Page

UML

Unified Modelling Language by OMG (Object-Modelling-Group). Graphical Modelling Language for SW Engineering/Development

WP

Working Page

XCP

Extended Calibration Protocol ASAM-MCD 1 (XCP)

XCPoE

Extended Calibration Protocol over Ethernet

XCT

XETK Configuration Tool used to configure the (some ETKs), FETK and XETK devices

## Conventions

The following typographical conventions can be used in this document:

| OCI_CANTxMessage msg0 = | Code snippets are presented on a gray background and in the Courier font. Meaning and usage of each command are explained by means of comments. The comments are enclosed by the usual syntax for comments. Changes are red marked. |
| --- | --- |
| Choose File Open. | Menu commands are shown in boldface. |  |
| Click OK. | Buttons are shown in boldface. |  |
| Press <Enter>. | Keyboard commands are shown in angled brackets. |  |
| The "Open File" dialog box is displayed. | Names of program windows, dialog boxes, fields, etc. are shown in quotation marks. |  |
| Select the file setup.exe | Text in drop-down lists on the screen, program code, as well as path- and file names are shown in the Courier font. |  |
| A distribution is always a one-dimensional table of sample points. | General emphasis and new terms are set in italics. |  |

# General usage of (F/X)ETK’s with different uC types

The ETK family comprises ETKs, XETKs and FETKs modules. Each of these modules offers a different set of functionality and performance.

In this document are all steps described how to integrate the ETK driver and how each function or feature works.

Following functions are mandatory for measurement and calibration :

- Handshake (like a login of ETK)

- Measurement (Distab17 and Trace)

- Calibration (Page Switch Mechanism with two page concept)

Additional features are :

- ACC (Advanced Code Check)

- Coldstart Measurement (Able to measure and calibrate before ignition on)

- CalWup (Calibration Wakeup – power supply of ECU)

In this version document is used a basis ETK driver version tag_20250205_A_XETKS20_S30_BRSS3_FETKS1_T1_TC2xxTC3xx_ExCode_V1_1_5.

NOTE:

In this document will be explained how to integrate the ETK driver in ECU software. When you need support or explanation of the function you can order an integration workshop or engineering hours.

NOTE:

It is assumed that the user is familiar with the use and handling of XCT Tool, HSP and INCA.

NOTE:

The define for used compiler is in the example code in the build_TC3xx_exampleCode_TASKING_V6.2.bat with -DTASKING_COMPILER. User can add also a define in one of the header files.

# ETK driver structure

The ETK driver has the normal uC OS with basis functions in the folder source.

In the subfolders for each uC type, like TC33xPD, is the information which target is selected and contains the file Target_Select.h, which is needed later in the ETK folder for compilation. This defines the uC type, class and if it’s a PD device.

The subfolder ETK contains all necessary functions and features in an own folder and the handshake files are directly in it. It’s recommended to copy all folders to your desired directory of your build and only compile what’s needed.

As additional hint for linking of some sections which are not used or unreferenced. In this case its helps to use for example with Tasking #pragma protect on  ….. #pragma restore.

# Handshake

After the microcontroller is running at the final PLL speed, all reset Pins are high (/PORST and /ESR0) and also VDD is high then ETK can start with the handshake procedure, which allows write and read access RAM and read access to FLASH. It’s like a login for uC.

## Needed Files

At first copy the folder ETK into your folder structure with following files :

Here a short description of the files :

CompileTime.c/.h is later for the advanced code check but is needed also for the handshake. Here its mandatory that the const for CODE_CHECK_PATTERN_DATA_DATE will be created.

Endinit.c/.h contains the security functions to get access to protected registers like unlock_wdtcon and lock_wdtcon. There is also activation of EMEM. When own functions are available this file can be replaced with self-created ones.

ETK_Integration_Cfg.h contains all defines for activation of features and functions, the register information’s and the calibration definitions.

ETK_SER_Handshake_Common.c is the general handshake sequence.

ETK_SER_Handshake_IFX.c is the specific sequence for IFX controllers.

ETK_SER_Handshake.h is the structure header for handshake.

GetTime.c/.h contain the time functions for calculation of timeouts and duration of handshake.

Target_Select.h is definition of CPU type and CPU class.

## Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file.

### CompileTime.c

#include "../a_std_type.h"  should be change to the file for standard types. Typically is used the name Platform_Types.h.

For later if ACC (advanced code check) will be used, which is recommended, then the ".ETK_Code_checkMemClass" must assigned to a address in linked which is located in the reference page (DATA FLASH).

group CAL_PARAM (ordered, run_addr=0xA0164000)

{

select ".rodata.CAL_PARAM";

select "*.CAL_PARAM";

select "*.measure_parameters";

select "*.ETK_Code_checkMemClass";

select ".rodata.*.CAL_PARAM";

}

### ETK_Integration_Cfg.h

Most important file for integration which is needed in each step where can be changed which function or feature is activated through integration.

#include "../a_std_type.h"  should be change to the file for standard types. Typically is used the name Platform_Types.h. Same as before in CompileTime.c.

Here now will be also linked the register file where all addresses of registers are linked. You can use the delivered .sfr file from ETAS TC39X_ts_MCSFR.sfr which works for all TC3xx uC or use the include from your own .sfr file

### ETK_SER_Handshake.h

#include "../a_std_type.h"  should be change to the file for standard types. Typically is used the name Platform_Types.h.

### GetTime.h

#include "../a_std_type.h"  should be change to the file for standard types. Typically, is used the name Platform_Types.h.

## Call of function

There are two types of handshakes. The initial handshake with all functionalities and the cyclic handshake if the initial was not successful.

The initial handshake must be called before the OS is running and after setting up the PLL. This will be called via command SER_Initial_Handshake_Execute();.

The cyclic handshake must be called in a runnable / raster with the recommended time cycle of 10ms and is called SER_Cyclic_Handshake_Check_And_Execute();.

For both handshake types the same header files are needed. Please add:

#include "ETK/ETK_Integration_Cfg.h"

#include "ETK/GetTime.h"

#include "ETK/ETK_SER_Handshake.h"

## A2L configuration in XCT Tool

When now the code is compiled and flashed the ETK must be configured that handshake is tried to be done from ETK side.

Best is to use a blank default configuration from XCT tool (4.4.3 Build 97) where you choose the uC and ETK type. Then set Handshake Timeout [ms] to -1 (infinity). When this is set, ETK tries to make a handshake as long as the handshake is successfully performed. This means he is accessing to the IOSR and COMDATA register in this time.

For example, I will use the BR_XETK-S3A and TC39x_ED device, because this supports all functions and features like TraceToRam.

When this configuration is downloaded then you can see after reset or power on the ECU the handshake is performed, and you have access to RAM and flash in hex editor.

# Measurement methods

After the handshake is done there are two different types for measurement, Distab17 and Trace. Both can work standalone and be combined to Trace with supplementary Distab.

## Distab17 method

Distab means Display Table and is a measurement method which gets an address list from INCA Experiment over Event Config Area (located in RAM and be written from ETK) and copies in a runnable the values into the Config Output Area (located in RAM and written by ECU), where the ETK takes the data and provide it back to INCA or third party tool.

### Needed Files

At first copy the folder D17 into your folder structure in the folder ETK with following files:

Here a short description of the files :

Distab17_Cfg.h is the configuration file where the maximum events and size of config and output area are defined. Also defines supplementary Distab.

Distab17_Def.h contains the base type definitions for all Distab based ECU routines.

Distab17_Fct.c/.h containing the function implementation to process a Distab event.

Distab17_Inc.h contains the type definitions for Distab data.

Distab17_Inst.c/.h containing the declaration of the Distab17 data structs. Relevant section for linker.

ETK_SER_Handshake.h is the structure header for handshake.

Distab17_Processes.c/.h - This function looks up the event configuration, decides whether this event is enabled, used TDM or the output table, if necessary invokes the data copy and finally executes the trigger.

### Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file.

#### ETK_Integration_Cfg.h

Activate the Distab17 measurement method by activating define DISTAB_SUPPORTED.

#### Distab17_Cfg.h

Here in general only two changes should be done. These are the sizes for the config area and output area size where the addresses and output values are located. This can be calculated manually by the formula:

4 Bytes header/event list + 4 Bytes/event + 24 Bytes header/raster + 4 bytes address/signal + signal size

When you have an available A2L file you also can select in INCA in VSD all measurements and then you get visualized how many bytes are selected. This you can use as estimated value for config and same for output area.

The default value for maximum number of Distab events is 255. With this number the eventlist has a size of 0x400 bytes. With reducing this number, the size of eventlist will also decrease. I recommend not to change that.

#### Distab17_Inst.c

All data structures needed for the ECU part of using the Distab17 are instantiated here. Important for Distab17 is the alignment of 4 Byte for all three different sections for the linker.

"ETK_DisTab17_EventList"

"ETK_DisTab17_EventConfigArea"

"ETK_DisTab17_EventOutputArea"

You can link them where you want. As recommendation for uC with EMEM it should be linked to the EMEM and the other can be linked to any location of SRAM or LMU/DLMU. Best is to create a group for Distab in linker file. In this example its located on SRAM from Core0 with offset from 0xC000 (0x7000C000). I advise also to set a fix value for this linker section that when some future changes will be done the addresses are not changing. So you can avoid to change the parameters in A2L file every new software build.

group distabTables (ordered, run_addr = mem:mpe:dspr0[0xC000])

{

select "*.ETK_DisTab17_EventList";

select "*.ETK_DisTab17_EventConfigArea";

select "*.ETK_DisTab17_EventOutputArea";

}

### Call of function

The Distab process will be called in each runnable the customer wants to get a Distab raster for measurement in INCA or in third party tool.

The Distab process must be called in a runnable / raster with a defined time cycle or synchronous time cycle and is called with a index beginning with 1 like Distab17_Process(1);. Best is to use the fastest raster with the lowest index number.

For example, you have three runnable’s with 1ms, 10ms and 100ms you set the calls like following:

FUNC(void, Xetk_CODE) Xetk_CyclicEvent1ms()

{

Distab17_Process(1);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent10ms()

{

Distab17_Process(2);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent100ms()

{

Distab17_Process(3);

}

You must add following header file which is needed in your runnable file. Please add:

#include "ETK/D17/Distab17_Processes.h"

### A2L configuration in XCT Tool

When now the code is compiled and flashed the ETK must be configured that Distab eventlist, config and output area is known for ETK. Also, the memory segment must be defined and the Distab runnable/raster created with definition of cycle time.

Now load the before created A2L file in XCT tool, open in parallel the .map file and search for the addresses for the Distab eventlist, config and output area.

First important part is that they must be located in a memory segment from type 	     VARIABLES RAM, because ETK must know where he can write and read.

Now go to the tab Distab Editor, activate Distab Version 17 and enter the addresses from map file for eventlist, config and output area start with each size. Then add the number of triggers depending on number of runnable/raster and assign the ECU ID (index number). I recommend using ECU ID = Trigger number = index of Distab process. Then it’s easier later for understanding and usage. At least add the data acquisition channel range for the used Distab raster. Here you can use in this case 1 to 3 but I recommend using 1 to 32 because when later more runnable/raster ´s are added then it’s easier to change.

Now go to the tab Raster Editor and add you runnable/raster ‘s with name, t in microseconds for cycle time, highest priority for fastest raster, Distab for acquisition control and trigger source you can choose a fixed Hardware Trigger or a Dynamic. I recommend Dynamic Trigger, then ETK detects which Trigger is correct one.

Then generate A2L file, merge A2L snippet with measurements. Now measurement in INCA should work. The ECU property for variables per second and bytes per second must not be zero for Distab17. If its zero, then TDM is used instead of Distab17 !!!

### Verify in XCT Tool and use in INCA

After import A2L in INCA, initialize and reconfigure your ETK then you can add variables to the defined raster.

To verify that you can connect in parallel with XCT tool and open with hex editor the eventlist, config and output area and see the configuration and the values changing. In this example I have added for each runnable/raster one 8 byte signal from Core1 (0x6000….).

## Trace based measurement method

Trace data is stored in the so-called Emulation Memory (EMEM) when TracToRam is only possible. By default, Emulation Memory consists of a regular memory array (TCM – Trace Calibration Memory) built out of RAM Blocks (Tiles), which can be split into a Trace memory and a Calibration memory part. When TraceToPin (FETK-T) is used then the whole EMEM can be used as Calibration Memory. When TraceToRam is used the first 512 Kbyte will be used as Trace Fifo.
For Trace is no event config and or output area necessary. This will be done by the Trace engine and unit inside EMEM. Only trace windows must be defined for ETK where the ETK takes the data and provide it back to INCA or third-party tool.

### Needed Files

At first copy the folder TRACE into your folder structure in the folder ETK:

Here a short description of the files :

Trace_Fct.c/.h contains the function to write the trace trigger.

Trace_Inst.c/.h containing the declaration of the trace data structs. Relevant section for linker.

### Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file.

#### ETK_Integration_Cfg.h

Activate the trace measurement method by activating define TRACE_SUPPORTED.

#### Trace_Fct.c

Here must two changes be done. #include "../a_std_type.h"  should be change to the file for standard types. Typically, is used the name Platform_Types.h. 
And comment the line for Distab17_Fct,h, because this is only needed when supplementary Distab is used.

#### Trace_Inst.c

#include "../a_std_type.h"  should be change to the file for standard types. Typically, is used the name Platform_Types.h.

"ETK_Trace_TriggerByValue"

You can link it where you want, and it should be 4 byte aligned. As recommendation for uC with EMEM it should be linked to the EMEM and the other can be linked to any location of SRAM or LMU/DLMU. I advise also to set a fix value for this linker section that when some future changes will be done the addresses are not changing. So, you can avoid changing the parameters in A2L file every new software build.

group TRACE_TRIGGER (ordered, run_addr = mem:mpe:dspr0[0x20])

{

select "*.ETK_Trace_TriggerByValue";

}

### Call of function

The trace trigger will be called in each runnable the customer wants to get a Trace raster for measurement in INCA or in third party tool.

The trace trigger must be called in a runnable / raster with a defined time cycle or synchronous time cycle and is called with the trigger number beginning with 1 like TraceExecTriggerByValue(1);. Best is to use the fastest raster with the lowest trigger number.

For example, you have three runnable’s with 1ms, 10ms and 100ms you set the calls like following:

FUNC(void, Xetk_CODE) Xetk_CyclicEvent1ms()

{

TraceExecTriggerByValue (1);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent10ms()

{

TraceExecTriggerByValue (2);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent100ms()

{

TraceExecTriggerByValue (3);

}

You must add following header file which is needed in your runnable file. Please add:

#include "ETK/ TRACE/Trace_Fct.h"

### A2L configuration in XCT Tool

When now the code is compiled and flashed the ETK must be configured for trace windows and where is the trace trigger.

Now load the before created A2L file in XCT tool with handshake, open in parallel the .map file and search for the address from trace trigger.

Now go to the tab Memory Layout Editor and add the trace windows with start address and size where your measurements are located. Then assign bus master/slave and enable if needed/wanted FGT (fine grain trace).

Now switch the tab to Raster editor and add for each trace trigger in a runnable/raster a new trace raster with cycle time, priority (for all same), Trace for acquisition control and new Trace for trigger source. Then change trigger address and value of runnable/raster.

Then generate A2L file, merge A2L snippet with measurements. Now measurement in INCA should work.

### Verify in XCT Tool and use in INCA

After import A2L in INCA, initialize and reconfigure your ETK then you can add variables to the defined raster. To check that all is working as expected you can open in parallel XCT tool and look over monitoring variable ETAS_XETK_TraceCurrent the actual trace data rate and see in hex editor on trace trigger address when trace trigger is written from ECU software. In this example trace trigger is located at 0x70000020.

## Supplementary Distab measurement method

This is a combination of trace based measurement with Distab and is needed when not all measurement variables can be measured via trace due to missing trace windows and this will be measured then via Distab, which is then located inside of a trace window. In this case only trace runnable/raster are defined.

### Needed Files

For this you need to copy both folder TRACE and DISTAB into your folder structure :

The description of the files you can find under 5.1.1 and 5.2.1.

### Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file.

#### ETK_Integration_Cfg.h

Activate the Distab and Trace measurement method by activating define DISTAB_SUPPORTED and TRACE_SUPPORTED.

#### Here you must do all the changes from Trace and Distab. Only difference here is that in the file Trace_Fct.c the #include "../D17/Distab17_Fct.h" must be active.

### Call of function

The supplementary Distab will be called over Distab process in each runnable the customer wants to get a Trace raster for measurement in INCA or in third party tool.

The Distab process must be called in a runnable / raster with a defined time cycle or synchronous time cycle and is called with an index beginning with 1 like Distab17_Process(1);. Best is to use the fastest raster with the lowest index number.

For example, you have three runnable’s with 1ms, 10ms and 100ms you set the calls like following:

FUNC(void, Xetk_CODE) Xetk_CyclicEvent1ms()

{

Distab17_Process(1);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent10ms()

{

Distab17_Process(2);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent100ms()

{

Distab17_Process(3);

}

You must add following header file which is needed in your runnable file. Please add:

#include "ETK/D17/Distab17_Processes.h"

### A2L configuration in XCT Tool

When now the code is compiled and flashed the ETK must be configured that Distab eventlist, config and output are is known for ETK. Also, the memory segment must be defined and the Distab runnable/raster must be created as Trace with definition of cycle time, configured as supplementary Distab and set the trace trigger for supplementary Distab.

Now load the before created A2L file in XCT tool where only handshake is configured, open in parallel the .map file and search for the addresses for the Distab eventlist, config and output area.

First important part is that they must be in a memory segment from type VARIABLES RAM, because ETK must know where he can write and read.

Now go to the tab Distab Editor, activate Distab Version 17 and enter the addresses from map file for eventlist, config and output area start with each size. Then add the number of triggers depending on number of runnable/raster and assign the ECU ID (index number). I recommend using ECU ID = Trigger number = index of Distab process. Then it’s easier later for understanding and usage. At least add the data acquisition channel range for the used Distab raster. Here you can use in this case 1 to 3 but I recommend using 1 to 32 because when later more runnable/raster ´s are added then it’s easier to change.

After that go to the tab Raster Editor and add your runnable/raster ‘s with name, t in microseconds for cycle time, priority (for all same), Trace for acquisition control and new Trace for trigger source. Then change trigger address for supplementary trace trigger and value of runnable/raster.

After generating A2L file please verify that in section for trace trigger the ECU_ID entry with 1024 is there. This is needed due to default Distab17 settings. If it’s missing please add it.

/begin TRACE_TRIGGER_CFG

TRACE_TRIGGER 0 0x70000024

CONFIG "MaxTriggerValue" "3"

ECU_ID 1024

/end TRACE_TRIGGER_CFG

### Verify in XCT Tool and use in INCA

After import A2L in INCA, initialize and reconfigure your ETK then you can add variables to the defined raster. To check that all is working as expected you can open in parallel XCT tool and look over monitoring variable ETAS_XETK_TraceCurrent the actual trace data rate and see in hex editor on trace trigger address for supplementary Distab when trace trigger is written from ECU software. In this example trace trigger is located at 0x70000024.

Then also you can have a look when variables are not located in a trace window if its measure via Distab. For this open hex editor and look on address where Distab config area and output area is.

# Calibration

The calibration with ETK works after the two page concept. The reference page is related to the FLASH and the working page is located in any SRAM/EMEM/LMU and is a copy of reference page where parameter/curves/maps can be changed/calibrated.

## Needed Files

At first copy the folder CALIB into the folder structure in the folder ETK with following files:

Here a short description of the files :

ETK_SER_Page_Ctrl_Fct.c/.h contains the structure and check of OMD, page switch mechanism, the activation of calibration and dis- and enbling of interrupts.

ETK_SER_Page_Ctrl_IFX.c is configuration of registers for calibration and EMEM tiles, if needed for ED-devices. This also contains the data cache invalidate function.

## Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file. In this case is in the added files nothing to change. Only in the

### CompileTime.c

Nothing mut be changed in the file..

Here you need two parts for calibration. The page switch mailbox and the OMD table.

"ETK_PageSwitch_MemClass"

"ETK_OMDTable_MemClass"

You can link it where you want, and it should be 4 byte aligned. As recommendation for uC with EMEM, where you want to store working page over ECU sleep or reset, it should be linked to the EMEM and the other (PD-devices) can be linked to any location of SRAM or LMU/DLMU. I advise also to set a fix value for this linker section that when some future changes will be done the addresses are not changing. So, you can avoid changing the parameters in A2L file every new software build.

group EmuRAM_CodeCheck_Pattern(ordered, run_addr=0xB93E0000)

{

select "*.code_checkMemClassEMURam";

select "*.ETK_PageSwitch_MemClass";

select "*.ETK_OMDTable_MemClass";

}

### ETK_Integration_Cfg.h

Activate the calibration method by activating define CALIBRATION_SUPPORTED. This can be activated independent from measurement.

### Call of function

The calibration process will be called in one runnable.

The page switch must be called only in one runnable / raster with a defined time cycle.  and is called with SER_ETK_PAGE_SWITCH_BY_ECU();. Best is to use a 10ms runnable / raster for that.

When the cached flash is used and you calibrate values on working page then this value will not be updated inside of the cache and this will have no influence on functions who are working with that. In this case the data cache must be invalidated, when no such function is available in the software. When this happens please call at the end of the task  (depends on how often this must be called,, I recommend 10ms) the data cache invalidate function via calling dataCacheInvalidate();.

For example, you have three runnable’s with 1ms, 10ms and 100ms you set the call in 10ms following:

FUNC(void, Xetk_CODE) Xetk_CyclicEvent1ms()

{

Distab17_Process(1);

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent10ms()

{

Distab17_Process(2);

if (SER_ETK_Check_PAGE_SWITCH_BY_ECU() == 1) // Page switch requested

{

SER_ETK_PAGE_SWITCH_BY_ECU();

}

dataCacheInvalidate();

}…

FUNC(void, Xetk_CODE) Xetk_CyclicEvent100ms()

{

Distab17_Process(3);

}

You must add following header file which is needed in your runnable file. Please add:

#include " ETK/CALIB/ETK_SER_Page_Ctrl_Fct.h"

### A2L configuration in XCT Tool

When now the code is compiled and flashed, the ETK must be configured that page switch mailbox and OMD table are known for ETK. Also, the memory segment must be defined which contains the page switch mailbox and OMD table.

Now load the before created A2L file in XCT tool, open in parallel the .map file and search for the addresses for the page switch mailbox and OMD table.

First important part is now to enter on layer Hardware Settings the mailbox address and OMD address with there settings.

The Version is 2, Autostart Behaviour is per default Last Active Page and Protocol Based page switch is used. MailboxVersion can only be 3 and for Mailbox Setup Time enter a value between 100ms and 500ms. When value 0 is used page switch will never work. This must be avoided. For MailboxAddress take the address from map file. Here ist 0xB93E0008. In this example code only OMD CID 2 is supported. As OMD version only 0 can be used.

For OMDAddress take the address from map file. Here ist 0xB93E00014. The maximum length comes from the size of OMDTable_MemClass with 0x1A4 = 420. In general 32 handles are used for calibration. But now I use the last part of EMEM for other things like OMD table, page witch mailbox etc. (Distab can be located there, ACC). So number of emulation handles is 31.

Second important part is on layer Memory Layout Editor that they must be located in a memory segment from type VARIABLES RAM, because ETK must know where he can write and read. In this case with EMEM it will be used the last part of it at address 0xB93E0000. This is important when ED devices are used that the working page stays over ECU reset or when its off to start from it.

Now also a DATA FLASH segement is needed which defines the reference page. CODE FLASH segments are not mandatory bur can be used for checksum calculation to compare if data in ECU from CODE section is same like in INCA.

Depending on calibration method for Infineon are two different methods available. More will be told in the integration workshop.

#### Calibration Method Fixed Size

This method is also needed when later the (F/X)ETK will be used in 3rd party usecase as XCP device. Here the handles can be defined on different locations, but they must all have the same size. Important thing is they are aligned to their size and may only include data areas that are also initialized. In this example is used the whole EMEM execpt the last part.

At first click on Fixed Size and Cretae a default handle set. Then change the original address where you want to locate your working page. The size must be the same for all calibration handles. The emulated address will be dynamically filled in INCA depending on which characteristics are added in a experiment.

#### Calibration Method Reconfigurable Size

This method can only be used with INCA. Here the user define one big block for working page with start address and size, where then later depending on size of data flash segments the handles can be defined with variable sizes. In this case the minimum alignment of a data flash segment must be 0x20. With that calibration method you can handle many different data flash segments with different sizes with only one definition and INCA calculates and configures dynamically the calibration handles.

Then generate A2L file, merge A2L snippet with measurements and characteristics. Now calibration in INCA should work.

### Verify in XCT Tool and use in INCA

After import A2L in INCA, reconfigure and initialize in the hardware configuration you should be able now to see, that checksum caluclation will be done depending on your memory segment definition for code, working and reference page. This you can verify in the MPM (memory page manager).

After that the experiment can be opened and characteristics can be added.

After switching the page in experiment you are able to change values in the calibration windows. Increase changes are displayed with red arrow and decreased values wqith blue arrow to the orriginal value. The Diff : 3 shows that overall 3 Bytes where changed. This can differ from number of changed variables depending onb byte size of each characteristic.

To verify the used and active calibration handles you can open the configuration over Variables  Limited Emulation RAM  Show Configuration

Then you see the type, number of handles and which data segment with size is mapped to which RAM location.

In parallel you can check the same in XCT with uploading the configuration and opening the hex editor. Now navigate to the page switch mailbox address with type variables.

If something is wrong or not working then access to register is not given, trusted functions / HSM are active or some flash is not intialized etc. Best is to begin here maybe with only one small handle to verify and check this function.

# Advanced Code Check (ACC)

The advanced code check feature makes sure that the used code and data in INCA is the same like in the flashed ECU. When this is different it doesn’t allows the user to calibrate to avoid inconsistent data between data in INCA and in the ECU.

## Needed Files

At first copy the folder ACC into the folder structure in the folder ETK with following files:

Here a short description of the files :

ETK_CodeCheck.c/.h contains the structure for ACC and cyclic copy process.

## Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file. In this case is in the added files nothing to change. Only in the

### ETK_CodeCheck.c

Nothing mut be changed in the file..

Here you need three parts for ACC. The CodeClass in Flash (defined before in handshake), MemClass in RAM and EMURAM. So now you need to assign to sections.

"code_checkMemClassRam"

"code_checkMemClassEMURam"

You can link it where you want, and it should be 4 byte aligned. As recommendation for uC with EMEM, the class for EMURam must be where you have the page switch method on EMEM and the class for Ram on any SRAM. LMU/DLMU.

group SRAM_CodeCheck_Pattern(ordered, run_addr=0x70000030)

{

select "*.IRAM";

select "*.ERAM";

select "*.code_checkMemClassRam";

….

}

group SRAM_CodeCheck_Pattern(ordered, run_addr=0xB93E0000)

{

select "*.code_checkMemClassEMURam";

select "*.ETK_PageSwitch_MemClass";

select "*.ETK_OMDTable_MemClass";

}

### ETK_Integration_Cfg.h

Activate the calibration method by activating define ACC_SUPPORTED. This can be activated independent from all other functions but should be done with calibration.

### Call of function

The ACC process will be called on two different section.

The initialization of ACC must be called directly before handshake and will be processed with initRAMCheckPattern();.

The cyclic called should be done in the same runnable / raster like the page switch method each 10ms and is called with CopyCodeCheckPattern_cyclic(0);.

For example, you have three runnable’s with 1ms, 10ms and 100ms you set the call in 10ms following:

FUNC(void, Xetk_CODE) Xetk_CyclicEvent10ms()

{

Distab17_Process(2);

if (SER_ETK_Check_PAGE_SWITCH_BY_ECU() == 1) // Page switch requested

{

SER_ETK_PAGE_SWITCH_BY_ECU();

}

CopyCodeCheckPattern_cyclic(0);

dataCacheInvalidate();

}…

### A2L configuration in XCT Tool

When now the code is compiled and flashed, the ETK must be configured that advanced code check is supported and activated. Important is, that the pattern in the code section is in a DATA FLASH memory segment and the RAM and EMURAM MemClass are located in a RAM VARIABLES segment.

Now load the before created A2L file in XCT tool, open in parallel the .map file and search for the addresses for the CODE_CHECK_PATTERN.

Now enter this on layer Hardware Settings in the correct lines and

In the lines Pattern Length is the length of pattern defined. Typically is used 8 Byte with 0x8 and for refresh Timeout the 20ms.

### Verify in XCT Tool and use in INCA

After import A2L in INCA, reconfigure and initialize in the hardware configuration you will see no change to before because all is working fine. Only when a mismatch in one of the addresses or patterns occur, then a popup window will be opened with the information, what is the reason. When this happens, the user cannot change values and calibrate. When user wants to calibrate, he must flash.

Issue with wrong pattern address :

Issue with wrong pattern :

After that information no checksum will be calculated and calibration is forbidden.

# Coldstart measurement

The coldstart measurement feature makes possible for the user to setup the measurement configuration without ignition of ECU and to halt the ECU in handshake till the ignition is on. This allows the customer to make a coldstart measurement. Needed Files

At first copy the folder CS into the folder structure in the folder ETK with following files:

Here a short description of the files :

ETK_SER_Coldstart.c/.h contains the structure for coldstart and  process with patterns during the handshake.

## Changes in Files for integration

Here is definition what must be adapted to ECU software for integration and what must be linked in linker file.

### ETK_SER_Coldstart.c

#include "../../a_std_type.h"  should be change to the file for standard types. Typically is used the name Platform_Types.h.

" ETK_ColdStart_MemClass"

You can link it where you want, and it should be 4 byte aligned. As recommendation put the Coldstart class to the Ram on first core CPU0.

group SRAM_CodeCheck_Pattern(ordered, run_addr=0x70000030)

{

select "*.IRAM";

select "*.ERAM";

select "*.ETK_ColdStart_MemClass";

select "*.code_checkMemClassRam";

….

}

### ETK_Integration_Cfg.h

Activate the coldstart method by activating define COLDSTART_SUPPORTED. This can be activated independent from all other functions but should be done with calibration.

### Call of function

The coldstart process will be executed during initial handshake process. For that is no separate call necessary.

### A2L configuration in XCT Tool

When now the code is compiled and flashed, the ETK must be configured that coldstart hand is supported and activated. Important is, that the Coldstart Class is located in a RAM VARIABLES segment.

Now load the before created A2L file in XCT tool, open in parallel the .map file and search for the first address of ETK_ColdStart_MemClass. In this case its 0x70004458.

Now enter this on layer Hardware Settings in the correct lines and

In the lines Wait and Ready Pattern there should be the same patterns like in the code used. Default values are 0x57 0x41 0x49 0x54 for wait pattern and 0x52 0x45 0x44 0x59 for ready pattern.

### Verify in XCT Tool and use in INCA

After import A2L in INCA, reconfigure and initialize in the hardware configuration you will see, that you are able to initialize when ignition is off and start measurement. You will see that INCA displays that ECU is off.

In the moment, when ignition is on immediately you can see values in INCA.

# Contact Information

##### Technical Support

For details of your local sales office as well as your local technical support team and product hotlines, take a look at the website: www.etas.com/hotlines

##### ETAS Headquarters

| ETAS GmbH |  |  |
| --- | --- | --- |
| Borsigstraße 24 | Phone: | +49 711 3423-0 |
| 70469 Stuttgart | Fax: | +49 711 3423-2106 |
| Germany | Internet: | www.etas.com |

Figures

Tables

Index

E

ETAS, 6

Revision History

The information in this chapter is tracked for internal purposes only and shall not be printed with the PDF. The revision history refers to the changes made in the document.

| Version, date author | Comment, Description |
| --- | --- |
| V0.1, … | Initial version |